# 1产品简介/Product Brief

全自动蛋液蛋壳分离机，由分离机配置相应蛋品梳理装置组成。蛋液蛋壳分离机先将输送 至分离工位的蛋敲破，然后通过特种刀片将蛋壳拨开，从而使蛋液流出，再经接蛋液托盘和  导流槽，将蛋液输送至相应的分离器内，通过分离器进一步将蛋清和蛋黄分离(分离器说明  见分离器说明书)。

The   automatic   egg   shell   separator   consists   of   a   separator   and   corresponding   egg   conveying   devices.The   egg   liquid   eggshell   separator   first   breaks   the   eggs   transported   to   the   separation   station,then  uses  a  special  blade  to  pry  open  the  eggshell,allowing  the  egg  liquid  to  flow  out. Then,through  the  egg  liquid  tray  and  guide  groove,the  egg  liquid  is  transported  to  the

corresponding   separator,and   further   separates   the   egg   white   and   egg   yolk   through   the separator(see the separator manual for instructions).

设备壳体采用304不锈钢制作，便于清洗。整体结构紧凑，占地面积小，产量适中。适合中小 型食品厂，糕点房等单位使用。

The   equipment   shell   is   made   of   stainless   steel(304),so   convenienting   to   clean.

The   overall   structure   is   compact,small   install   area,and   the   output   is   moderate.

Suitable  for  small  and  medium  food  factory,pastry   shop  and  other  units.

2  产品技术参数/Product   Specifcations

产品名称/Name:   全自动蛋液蛋壳分离机

Automatic     EggBreaker     machine

额定电压/Voltage:AC90V~260V 额定频率/Frequency:50/60Hz

额定功率/Power:300W

气源/ASUP:0.6Mpa         10L/min

产品型号/Model:EB-I-2.7K

最佳分离速度/Speed:2700eggs    pcs    hour

产品外形尺寸/Overall    size:1750*1150*1050mm 重量/Weight:110~130Kg

安置场地尺寸/Install      area       size:2000*1500*1050mm 产品型号/Model:EB-II-5.4K

最佳分离速度/Speed:5400eggs    pcs    hour

产品外形尺寸/Overall    size:1750*1150*1050mm 重量/Weight:120~140Kg

安置场地尺寸/Install area size:2000*1500*1050mm

产品型号/Model:EB-III-8.1K

最佳分离速度/Speed:8100 eggs pcs hour

产品外形尺寸/Overall    size:1750*1150*1050mm 重量/Weight:140~160Kg

安置场地尺寸/Install      arca       size:2000*1500*1050mm

产品型号/Model:EB-IV-10.8K

最佳分离速度/Speed:10800eggspcs       hour

产品外形尺寸/Overall    size:1750*1150*1050mm 重量/Weight:150~170Kg

安置场地尺寸/Install      arca       size:2000*1500*1050mm

3 产品工作环境/Product Work Condition

1 ) 工 作 环 境/work      condition: 温 度+ 1 0 °~+ 6 5 °相 对 湿 度9 0 %以 下 。

Temperature    10~+65°relatively   moisture   below    90%.

2 ) 贮 存 环 境 /stock     condition: 温度 - 25°~+65°相对湿度90%以下。 Temperature   -25~+65°relatively    moisture   below    90%.

##### 4  整机示意图/Product Layout

大托盘安装座

盛蛋托盘 Egg tray

托盘支撑杆

Egg tray supporter

左前门

Left front door

气源输入 Gas input

电源输入

Power input

残蛋液承接托盘

Lost egg liquid collection tray

上罩壳

Uper cover   shell

敲蛋信号采集器

Signal  collector  ofknock  egg

右前门

Right front door

蛋液承接托盘

Egg liquid collection tray

敲蛋弹簧

蛋壳出口

Eggshell outlet

砖蛋液彼导 ua         uider

控制面板

control panel\

主机架

Main frame

蛋品固定器 Egg  ixator

脚轮组件

casters

常规蛋壳蛋液分离机

Conventional eggshell and egg liquid separator

蛋品固定器

摆r 托n  gg  try  fixing  seatEgg  fixator

摆蛋托盘

托盘支撑杆

Egg tray supporter

左前门

Left front door

气源输入 Gas input

电源输入

Power input

残蛋液承接托盘

Lost egg liquid collection tray

上罩壳

敲蛋信号采集器

Signal collector of knock egg

敲蛋弹簧

Spring for Knock Egg

蛋壳出口

Eggshell outlet

Se    eg    nquid    guider

控制面板

control  panel

脚轮组件

casters

主机架

Main frame

摆蛋托盘式蛋壳蛋液分离机

Arraying    egg    tray    type    eggshell    and    egg    liquid    separator

##### 5  产品组装说明/Product lostall Method

拆箱取出部件名称如下：托盘，电源线和脚轮及附件(可能会有翻斗分离器参考分离器说明书)。 Unpack   and  remove  the   components  with  the   following  names:tray,power   cord,casters,and  accessories(there  may  be   a  tipping  bucket   separator,please  refer  to   the  separator  manual).

按照上图的相应位置进行组装，顺序如下：

Plcase refer to  above  layout  pic  to  install  the  machinc,steps  as  fllow:

1)将脚轮安装到支撑腿侧面，然后用扳手锁紧，并锁止刹车脚轮。

Install the casters on the  side of the  support legs,then lock them with a wrench and lock the brake   casters.

2)如果所选用设备为常规蛋液蛋壳分离机，鸡蛋托盘安装如下图。先将托盘固定螺钉和鸡蛋托盘支   撑杆自由端固定螺钉取出，然后将鸡蛋托盘整体靠近主机，在对应位置将支撑杆和主机通过螺栓连  接。该螺栓不可旋转太紧，然后将托盘按照下图旋转方向拉起，将托盘固定耳放置在托盘固定耳安  装座上，并用螺钉固定。注意固定耳的方向如图一所示为正确方向。完成后进入下一步调整挡蛋杆， 调整挡蛋杆完成后需将托盘支撑杆两端的螺栓拧紧。

If the  selected  equipment  is  a  conventional  egg-shell  separator,the  installation  of  the  egg  tray is shown below.First remove the fixing screws of the tray and the free end of the support rod

of the egg tray,then bring the entire egg tray close to the main machine and connect the

support  rod  and  the  main  machine  with  bolts  at  the  corresponding  position.The  bolt  should

not be  tightened  too  tightly,and  then  pull  up  the  tray  according  to  the  rotation  direction  shown below.Place the fixing ears of the tray on the mounting base of the fixing ears and secure them with  screws.Note that the  direction  of the  fixing  ears  is  shown  in  Figure  l  as the  correct

direction.After  completing  this  step,proceed  to  the  next  step  of  adjusting  the  egg-stopping

rod.After  adjusting  the  egg-stopping  rod,tighten  the  bolts  on  both  ends  of  the  tray  support  rod.

托盘固定螺钉

Tray retaining screws

旋转方向

Direction of rotation

支撑杆螺钉安装孔

支撑杆固定螺钉

Support rod screw

Support rod screw mounting holes

3)如果所选设备为摆蛋式托盘蛋壳蛋液分离机，安装方法如下图所示。将摆蛋式托盘末端拉手拉起， 支撑杆会跟随运动至其末端定位卡会自动卡在横梁上，摆蛋大托盘会在限位柱的位置停止翻转。此  时若支撑杆末端定位卡没有卡在横梁上，需人工干预将其卡在横梁上，然后在支撑杆和托盘连接处  的可调连接板处于松弛状态，需在尽可能上提托盘，同时下压支撑杆，从而去除支撑杆与托盘支撑  的活动间隙，此时将可调连接板上的螺栓拧紧。 (翻转时需注意侧壁的线管和气管没有被阻挡)

If the  selected  equipment  is  a  swing  egg  tray  eggshell  liquid  separator,the  installation  method is  shown in the  following  figure.Pull up the handle at the end of the egg laying tray,and the

support  rod  will  follow  the  movement  until  its  end  positioning  card  automatically  gets  stuck on the  crossbeam.The  egg  laying  tray  will  stop  flipping  at  the  position  of the  limit  column.

If the positioning card at the end of the support rod is not stuck on the crossbeam at this time, manual  intervention  is  needed  to  clamp  it  on  the  crossbeam.Then,the  adjustable  connecting plate at the connection between the  support rod and the tray is in a relaxed  state.It is

necessary to lift the tray as much as possible and press down on the support rod to remove the active  gap  between  the  support  rod  and  the  tray  support.At  this  time,tighten  the  bolts  on  the adjustable  connecting  plate.(When  flipping,it  is  important  to  note  that  the  tubing  and  trachea on  the  sidewalls  are  not  obstructed)

限位柱

Limiting   column

连接板

Connecting  plate

横梁  Beam

把手

Handle

定位卡

Positioning card

固定螺母

Retaining nut

4)在气源输入位置连接相应的气源管(φ8mm,0.6Mpa,10L/min) 并确保气源已打开。

Connect  the  corresponding   air   source  pipe(diameter   8mm,0.6Mpa,101/min)at   the  input position of the air source and ensure that the air source is turned on.

5)将电源线快速插头插入电源输入位置，另一端为防漏电保护插头(输入为AC90V—260V), 将其插 接供电口(必须有接地线)即可。

根据具体要求可能会配置不同国家标准的转换插头，若果配有相应的转化插头，需将转换插头先插 接在防漏电保护插头上，再插接供电口(必须有接地线)。

完成以上操作后需检查漏电保护插头上绿色的指示灯是否点亮。

如果亮起，按下T键，指示灯会熄灭，再次按下O 键指示灯会点亮； 如果没有熄灭，反复操作后无果，请联系厂家。

再设备没有完成检查工作前，先将按下T键确保电路断开。

Insert the quick plug of the power cord into the power input position,and the other end is the leakage

protection plug.The input voltage is AC90V-260V,and plug it in the power supply port(Must have the ground wire).

According to specific requirements,different national standard conversion plugs may be configured.If equipped with corresponding conversion plugs,the conversion plugs need to be plugged in the leakage protection plug and then plug into the power supply portt(Must have the ground wire).

After completing the above operations,it is necessary to check whether the green indicator light on the leakage protection plug is light.

If it lights up,press the Tkey,the indicator light will go out,and press the Okey again to light up the indicator light.

Ifit does not turn off and repeated operations have no effects.Please contact the manufacturer.    Before the equipment completes the inspection work,press the Tkey to ensure that the circuit is disconnected.

6)检查接蛋液盒是否安装正确，若果安装存在脱钩现象需将其取下进行再次安装。安装方法如下： 向上轻轻托起接蛋液盒，向左侧滑动，即可取下，安装时需将接蛋液盒两侧的缺口位置对照相应  的螺柱并向上托举，然后向右侧滑动，让侧边上的挂钩，挂在相应的螺柱上即可。(如下图所示)

Check  if  the  egg  box  is  installed  correctly.If  there  is  any  detachment  during  installation, remove  it  and  reinstall  it.The  installation  method  is  as  follows:

Gently lift the egg receiving box upwards and slide it to the left to remove it.

When  installing,compare  the  gap  positions  on  both  sides  of the  egg  receiving  box  with  the   corresponding  onesLift the bolt upwards  and  slide  it  to  the right,so that the hook  on the  side can  be  hung  on  the  corresponding  bolt.(As  shown  in  the  following  figure)

拆卸方向 Detach

安装方向 Install

托起 Lift

7)取两枚蛋(常用蛋)放置在靠近大托盘出口的蛋杯内，如图所示，然后将校准板放置在两枚蛋表 面，观察挡蛋杆的高低，确保挡蛋杆自由状态下，挡蛋杆下表面与校准板上表面贴合，若果位置不 合适可将挡蛋杆两侧固定板固定螺钉松开进行上下调整，调整过程中挡蛋杆尽量靠近大托盘出口。

Take two  eggs(commonly used  eggs)and  place them  in  the  egg  cup  near  the  outlet  of the  egg tray,as  shown  in  the  figure.Then,place  the  calibration  plate  on  the  two  egg  top.

observe the height of the egg blocking rod to ensure that the lower surface of the egg blocking rod is in a free  state and adheres to the upper  surface of the calibration plate.If the position is  not correct It is appropriate to loosen the fixing screws on both sides of the egg guard and

adjust  it  up  and  down.During  the  adjustment  process,the  egg  guard  should  be  as  close  to  the outlet  of the  large  tray  as  possible.Take  two  eggs(commonly  used  eggs)and  place  them  in

the  egg  cup  near  the  outlet  of the  large  tray,as  shown  in  the  figure.Then,place  the  calibration plate  on the two  egg tablesFace,observe the height of the  egg blocking rod to  ensure that the lower surface of the egg blocking rod is in a free state and adheres to the upper surface of the  calibration  plate.If the  position  is  not  correctIt  is  appropriate  to  loosen  the  fixing  screws  on

both  sides  of the  egg  guard  and  adjust  it  up  and  down.During  the  adjustment  process,the  egg guard should be as close to the outlet of the large tray as possible.

8)摆蛋式敲蛋机的挡蛋杆和普通敲蛋机挡蛋杆安装方法不同，如下图所示，校准方法相同。请参考 上一条说明。

The installation method of the  egg blocking rod of the  egg laying machine is  different  from that  of  a  regular  egg  knocking  machine,as  shown  in  the  following  figure.The  calibration

method  is  the   same,please  refer  to  the  previous  explanation.

挡蛋杆固定板

Egg blocking rod fixing plate

挡蛋杆

Egg blocking rod

校准板

Calibration   board

挡蛋杆固定螺钉

Egg blocking rod fixing screw

常规蛋壳蛋液分离机

Conventional  eggshell  and  egg  liquid  separator

挡蛋杆固定螺钉

Egg blocking rod fixing screw

挡蛋杆ocking rod

校准板

Calibration   board

挡蛋杆固定板

Egg blocking rod fixing plate

摆蛋托盘式蛋壳蛋液分离机

Arraying egg tray type eggshell and egg liquid separator

6,产品试机操作说明/Product trial operation instructions

1)确保主机内无异物后将漏电保护插头上的o键按下接通电源，指示灯点亮。同时设备控制面板处 绿灯点亮，如果控制面板绿灯没有点亮，需检查急停开关是否处于断电状态(急停按钮被按下), 需顺时针旋转急停按钮，将其解锁电源接通。如果此操作无果，请联系厂家。

After  ensuring  that there  are  no  foreign  objects  inside  the  host,press  the  o  button  on  the leakage protection plug  to  turn  on  the  power,and  the  indicator  light  will  light  up.

At the  same time,at the  device  control panel the green light is on.If the green light on the

control panel  is not  light,check  if the  emergency  stop  switch  is  in  a  power-off state  (the

emergency  stop  button  is  pressed),Rotate  the  emergency  stop  button  clockwise  to  unlock  and power  on.If  this   operation   is  unsuccessful,please   contact  the  manufacturer.

2)旋转启动旋钮主机开始工作，内部破蛋的零部件开始发出具有节奏的卡塔卡塔声响。反向旋转旋 钮，设备停止工作，但指示灯依然点亮。 (如果是摆蛋到托盘式蛋壳蛋液分离机，摆蛋托盘会自动 转动，没有转动的话需要将调速旋钮顺时针旋转，开启转动。注：顺时针旋转增大速度，逆时针旋 转降低速度)

The  main  engine  started  working  by  rotating  the  start  knob,and  the  internal  broken  parts

began  to  emit  rhythmic  katakata   sounds.Rotate  the  knob  in  reverse,the  device   stops  working, but the  indicator  light  remains  on.(If it  is  a  tray  type  egg  shell  and  egg  liquid  separator,the

egg  tray  will  automatically  rotate.If  it  does  not  rotate,you  need  to  turn  the  speed  controlknob clockwise   to   turn   it    on.Note:clockwise   rotation   increases    speed,counterclockwise   rotation

decreases    speed)

提速

Speed up

降速 Speed reduce

3)先在鸡蛋托盘的导流筒内分别放置4-10枚蛋。再放置第一枚蛋的时候，需用手再挡蛋杆前阻挡鸡 蛋，其目的是防止鸡蛋撞破并确保蛋品是横放的状态，然后依次放入多枚鸡蛋并确保导流筒内鸡蛋 横放滚落的状态。(如图所示)

常规机托盘内鸡蛋为自由滚落的方式，蛋品自行落入对应蛋杯中，蛋品在进入流蛋筒时容易堆积， 需人工干预，以确保蛋品排列进入流蛋筒。

摆蛋式蛋壳蛋液分离机托盘内靠托辊向前推动的方式将蛋品输送至蛋杯中。如果托盘内蛋品供应不 足或托辊运送速度过快会出现蛋品非横向进入流蛋筒，从而造成蛋品进入蛋杯时容易挤破。需将托 辊调速旋钮逆时针转动，降速或暂停运送。如果托盘内蛋品供应过多或托辊运送速度过慢会出现蛋 品在进入导流筒时堆积，需人工干预。确保蛋品横向进入导流筒。

First,place  4-10eggs  in  the  draft  tube  of  the   egg  tray.When  placing  the  first  egg,use  your

hand  to  block  the  egg  rod  in  front  of  it  to  prevent  the  egg  from  breaking  and  ensure  that  the

eggs   are   placed   horizontally.Then,place   multiple   eggs   in   sequence   and   ensure   that   the   eggs in  the   draft  tube   are  placed  horizontally   and  roll   down.(As   shown  in  the   figure)The  eggs  in

the  conventional  machine  tray  fall  freely  and  fall  into  the  corresponding  egg  cups.The  eggs

tend  to   accumulate  when   entering  the   egg   flow   tube,requiring  manual   intervention  to   ensure   that  the  eggs  are  arranged  and  fed  into  the  egg  flow  tube.The  egg-type  egg  shell  and  egg  liquid separator  uses  a  roller  to  push  the  eggs  forward  into  the  egg  cup.If  the  supply  of  eggs  in  the

tray  is  insufficient  or  the  roller  is  moving  too  fast,the  eggs  may  not  enter  the  flow  tube

horizontally,causing   them   to   break   when   entering   the   egg   cup.The   speed   controlknob   of   the roller  needs  to  be  turned   counterclockwise   to  slow  down  or  pause   the  delivery.If  there  is  too  much  supply  of  eggs  in  the  tray  or  the  roller  is  moving  too   slowly,the  eggs  may  accumulate

when   entering   the    flow   tube,requiring    manualintervention.Ensure   that    the   eggs    enter   the flow   tube   horizontally.

4)完成上述操作后，在蛋壳出口处，蛋液承接托盘出口以及残蛋液导流槽出口处分别放置相应的收 集容器。

After   completing   the    above    operations,corresponding   collection    containers   are    placed   at    the  outlet  of  the  eggshell,the  outlet  of  the   egg  liquid  receiving  tray,and  the  outlet  of  the  residual egg  liquid  flow  channel.

5)旋转开关旋钮，设备将自动工作，注意观察挡蛋杆位置是否有蛋被挤破，或者有蛋被卡在挡蛋杆 无法滚落，如果偶尔出现上述情况为正常，需人工处理，如果连续出现上述情况，请确认是否正确 调整挡蛋杆。在通过调整挡蛋杆无法避免上述情况，请联系厂家。

Rotate   the   switch   knob,and   the    equipment   will   automatically   work.Pay    attention   to   whether there  are  eggs  squeezed  or  stuck  in  the  position  of  the  egg  blocking  rod  that  cannot  roll  off.

If  the   above   situation   occasionally   occurs,it   is   normal   and   manual   handling   is  required.If  the above   situation   continues,please   confirm   whether   the   egg   blocking   rod   is   adjusted   correctly.  If  the  above  situation  cannot  be  avoided  by  adjusting  the  egg  shift  lever,please  contact  the

manufacturer.

6)设备运转后，需观察蛋品固定器位置鸡蛋的分离情况。

正常情况下鸡蛋运转至蛋品固定器下方，气缸会挤压固定柱，将鸡蛋固定，不能上下跳动，并开始 蛋壳蛋液分离工作，蛋壳会呈现出完整的两个半壳，蛋液会在落入蛋液承接托盘内并流出。

如果在蛋品在运送至固定器下方时，蛋品被挤破，说明蛋品固定片过低，则需要将固定片抬高。

如果在蛋品在运送至固定器下方，破壳的时候出现，蛋品跳起，说明蛋品固定片过高，无法固定蛋 品，需将固定片降低。

调整方法如下：关闭气源，逆时针转动调整螺钉，固定片降低，顺时针转动螺钉，固定片抬高。 调整幅度需根据所用蛋品的大小决定。

若果通过微调无法解决上述问题，则需要调整蛋品固定器整体的高低，蛋品固定器两侧通过安装片 与主机固定，可将固定螺钉松开后，整体上提或下压可大幅度调整固定片的高低。(结构如下图)

After  the  equipment  is  running,it  is  necessary  to  observe  the  separation  ofeggs  at  the  position of the  egg  holder.Under  normal  circumstances,when  the  egg  runs  below  the  egg  stabilizer,

the cylinder will compress the fixing column to fix the egg without jumping up and down,

and the  egg  shell  and  egg  liquid  separation  work  will  begin.The  egg  shell  will  present  two

complete half shells,and the  egg  liquid  will  fall  into  the  egg  liquid  receiving  tray  and  flow  out. If the  egg  is  squeezed  under  the  stabilizer  during  transportation,it  indicates  that  the  egg  fixing plate is too low and needs to be raised.

If the  egg  breaks  during  transportation  to  the  bottom  of  the  fixator,causing  the  egg  tojump,it indicates that the egg fixing plate is too high to fix the egg and needs to be lowered.

The  adjustment  method  is  as  follows:turn  off  the  air  source,turn  the  adjustment  screw

counterclockwise  to  lower  the  fixing  plate,turn  the  screw  clockwise  to  raise  the  fixing  plate. The adjustment range needs to be determined based on the size of the egg used.If the above

problem  cannot  be  solved  through  fine-tuning,it  is  necessary  to  adjust  the  overall  height  of

the egg holder.The two  sides  of the egg holder are fixed to the host through mounting plates.   After  loosening  the  fixing  screws,the  overall  height  of the  fixing  plate  can  be  greatly  adjusted by  lifting  or pressing  it up  or  down.(Structure  as  shown  in  the  figure  below)

浮动柱

Piston column

微调螺钉

Fine adjustment screw

固定螺钉

Fixed screws

蛋品固定板

Egg fixing plate

7)设备正常工作情况下，观察开蛋情况是否理想。如果通过调整蛋品固定器仍然无法避免蛋品被挤 破或蛋品跳跃的现象，需要调整敲蛋信号采集器的位置，该采集器正面有两颗螺钉，松动两颗螺钉 后，信号采集器固定架可进行旋转调整，向右旋转信号会提前响应，可解决蛋品跳跃的现象，向左 旋转信号会延迟响应，可解决蛋品被挤破的现象。以上操作仅仅是微调信号采集的响应时机。(如 图示)

Under  normal  operation  of  the  equipment,observe  whether  the  egg  opening  situation  is  ideal. If the  egg  is  still  unable  to  avoid  being  squeezed  orjumping  by  adjusting  the  egg  holder,it  is

necessary  to  adjust  the  position  of  the  egg  knocking  signal  collector.There  are  two  screws  on   the  front  of  the  collector.After  loosening  the  two  screws,the  signal  collector  holder  can  rotate and  adjust.Turning  the  signal  to  the  right  will  respond  in  advance,which  can  solve  the

problem  of  eggjumping.Turning  the  signal  to  the  left  will  delay  the  response,which  can  solve the  problem  ofegg  being  squeezed.The  above  operations  are  only  for  fine-tuning  the  response timing  of  signal  acquisition.(As   shown  in  the  diagram)

固定螺钉

Fixing screw

信号采集器固定架

Signal collector mounting bracket

7产品清洁与维护/Product elean and maintenance

1)清洗前需拔下插头，确保切断电源。

Make  sure  cut  down  the  electricity  and  unplug  before  clean  process.

2)打开主机上罩壳固定搭扣，将其掀起并折叠，放置在防脱块上。

Open the  fixing buckle  of the  cover on the main unit,lifting it up  and  folding it,and then put it  on  the  preventing  slipping  blocks

3)打开主机右前门，取出接蛋液托盒，单独清洗，并擦拭干燥。

Open the right front door and take out the egg liquid tray to rinsing,and then wiping it with dry   towel.

4)可以用喷枪冲洗主机内的蛋杯，破蛋开壳部件等，也可逐一用湿毛巾擦拭。擦拭和冲洗一部分后 可以接通电源，让其转动，待转动后再将电源切断(请务必切断电源)。继续擦拭和清洗其他部

件，直至清洗完毕后，将其擦拭干燥。(摆蛋托辊即蓝色塑料部位不可用高压水枪冲洗)

You can use the spray gun to rinsing the egg tray in the main engine and the parts of breaking 	the  egg  shell,or  wiping  them  with  a  wet  towel.After  wiping  and  washing  some  of  the  parts, you  can  turn  on  the  powe,making  machine  running  to  reveal  the  parts  unwashed.And  then

turn  off the  power,to  continue  wiping  other  parts  until  the  cleaning  is  finished.(Make  sure  the power is  cut  off before wiping parts.)

5)请勿向开关，电源以及后罩壳扣合缝隙内冲水。

Do not rinse the  switch,power  supply  and  the  gap  of the  rear  cover

6)定期向塑料轴承(开刀轴承每天涂抹)工件内涂抹凡士林或类似润滑脂。涂抹时需切断电源进行 操 作 。

Regularly  apply  Vaseline   or  similar  lubricating  grease  to   the  plastic  bearings(cutting

bearings  applied  daily)inside  the  workpiece.When  applying,cut  off  the  power   supply  for operation.

开刀轴承

Cutting    bearings

抬刀轴承

Bearing  of  lifting  knife

防链条跳齿器

Anti  chain  jumping  gear

刀架缓冲器

Holder  buffer  of  Knife

链条张紧块

Chain   tensioner

7)定期检查蛋杯链条张紧情况，链条在使用过程中会出现延长现象，需定期将链条张紧防止链条跳 齿现象发生。

Regularly  check  the  tension  of  the  egg  cup  chain.The  chain  may  experience  elongation during  use,and  it  is  necessary  to  regularly  tighten  the  chain  to  prevent  tooth  jumping.

8)定期检查抬刀轴承情况(如上图所示)。在刀片闭合的情况下用手拨动抬刀轴承，若果能拨动说明 运转正常，若果无法拨动或拨动存在阻力，则需将刀架缓冲器向刀架方向调整，使得抬刀轴承能轻

松转动，调整幅度不可过大，否则会影响刀片上升的高度，从而影响敲蛋效果。

调整方法：将缓冲器螺母松开缓冲器顺时针旋转，靠近刀架，逆时针旋转远离刀架。

Regularly  inspect  the  condition  of  the   knife  lifting  bearing(as  shown  in  the  above  figure).

When   the   blade   is   closed,manually   move   the   lifting   bearing.Ifit   can   be   moved,it   indicates

normal   operation.Ifit   cannot   be   moved   or   there   is   resistance   during   movement,the   buffer   of  the  tool  holder  needs  to  be   adjusted  towards  the   direction   of  the  tool  holder,so  that  the   lifting bearing   can  rotate   easily.The   adjustment  range   should  not  be  too   large,otherwise   it  will

affect  the  height  of  the   blade   and   affect  the  knocking   effect.Adjustment  method:Loosen  the buffer  nut   and   rotate   the   buffer   clockwise,close   to   the   tool   holder,and   counterclockwise

away  from  the  tool  holder.

9)定期检查敲蛋弹簧情况(如上图所示)。该弹簧为易损件，挡敲蛋力度较弱的情况下可适当调整 弹簧后端的螺母，顺时针旋转可增大敲蛋力度，在调整该螺母时需注意刀架被拉动后弹簧固定螺母 和定位套内是否有间隙，最大调整量取决于该间隙的大小，当调整后没有间隙需更换弹簧，不可过 度调整，过度调整螺母造成容易造成敲蛋刀组件损坏。

Regularly  check  the  condition  of  the   egg  knocking  spring(as  shown  in  the  above  figure).

This  spring  is  a  vulnerable   component.If  the  knocking   force  is  weak,the  nut   at  the  back   of  the spring   can   be   adjusted   appropriately.Turning   it   clockwise   can   increase   the   knocking   force.

When  adjusting  the  nut,attention  should  be   paid  to  whether  there  is  a  gap  between  the  spring fixing  nut  and  the  positioning  sleeve  after  the  tool  holder  is  pulled.The  maximum  adjustment amount  depends  on  the  size  of  the  gap.When  there  is  no  gap  after  adjustment,the  spring

needs  to  be  replaced   and   excessive   adjustment   is  not   allowed.Excessive   adjustment   of  the nut  can  easily  cause  damage  to  the  egg  knocking  knife  component.

必须保证有可压缩距离

已经没有有可压缩距离

No  compressible  distance

Must ensure compressible distance

8产品故障检查/Product   malfunction   inspection

1)如发现漏电故障后需检查接地线是否确保接地。

If  leakage  fault  is  found,it  is  necessary  to  check  whether  the  grounding  line  is  grounded.

2)蛋品未敲破现象原因如下：

一、敲蛋弹簧力度较小适当调整螺母。 (顺时针将其力度加大，反之则减小)

二、敲蛋弹簧失去弹性或断裂，需更换弹簧。

三、蛋品定位器，未固定蛋品。

首先需检查气源情况，是否未供气。

其次检查气缸导柱是否动作。

如果气缸没有动作，气源没有问题，可能是继电器、电磁阀或传感器

损坏，需更换电磁阀、继电器或传感器。在判断以上部件前前需检查电源转换器是否损坏，如果电 源转换指示灯(绿色)未点亮，说明电源转换器损坏需更换电源转换器。

如果气缸动作正常需检查蛋品固定器固定片是否上下浮动正常(正常情况是断气的状态下固定片可 轻松上下浮动),若阻力较大说明其关联的浮动柱孔内存在异物，须将浮动柱取出将浮动柱表面及 浮动柱孔用干毛净擦拭。

四、刀片位置过低，可适当将刀片提高。

五、刀架固定轴承磨损，间隙较大，需更换轴承。

The reasons for the phenomenon of eggs not being broken are as follows:

1、Adjust the nut appropriately when the force of the egg tapping spring is small. (Increase the force clockwise,otherwise decrease it)

2、The egg knocking spring loses elasticity or breaks,and the spring needs to be replaced.

3、Egg   locator,unfixed  egg.

Firstly,it is necessary to check the condition of the gas source to see if there is no gas supply.

Next,check if the cylinder guide pillar is moving.

If the cylinder does not move and there is no problem with the air supply,it may be due to damage to the relay,solenoid valve,or sensor,which needs to be replaced.  Before determining the above components,it is necessary to check if the power

converter  is  damaged.If  the  power  converter  indicator  light(green)is  not  lit,it indicates that the power converter is damaged and needs to be replaced.

If the cylinder operates normally,it is necessary to check whether the fixed piece of    the egg stabilizer is floating  up and down  normally(under  normal  circumstances,the fixed piece can easily float up and down when the air is cut off).

If the resistance is high,it indicates that there are foreign objects in the associated

floating column hole.The floating column must be removed and the surface and holes of the floating column should be wiped with a dry cloth.

4、The blade position is too low,and the blade can be raised appropriately.

5、The fixed bearing of the knife holder is worn and has a large gap,and the bearing needs to be replaced.

刀片(分左右)

Knife

刀柄复位弹簧  Sliding bearing

过动  BearinM⁰1-12)

开壳轴承(61801)

Open shell bearing

抬刀e钟 1 6 1earingofknife       older

刀架固定轴承 (SKFL001) Knife holder fixed bearing

3)设备无法正常工作时，需首先确认设备机械部分是否正常。

首先判断传动凸轮部分是否出现严重磨损，如果磨损严重需更换传动凸轮。

其次判断开刀轴承是否出现严重磨损，若果磨损严重需更换刀柄。

再次判断刀架轴是否出现断裂，如果断裂需更换刀架。

最后需打开后盖，判断传动齿轮是否出现滑齿现象，以及黑色驱动器指示灯是否出现红灯报警。 出现上述故障需拍照或视频发送厂家进行判断。

When  the  device  cannot  function  properly,it  is  necessary  to  first  confirm  whether  the mechanical part  of the  device  is  normal.

Firstly,determine  if  there  is  severe  wear  on  the  transmission  cam.If  the  wear  is  severe,

replace  the  transmission   cam.Secondly,determine  whether  there   is  severe  wear   on  the

cutting  bearing.If  the  wear  is  severe,replace  the  tool  handle.Check  again  if  the  tool  holder shaft  is  broken.If  it  is,replace  the  tool  holder.

Finally,it is necessary to open the back cover and check if there is any slipping of the

transmission  gear,as  well  as  whether  the  black  driver  indicator  light  shows  a  red  light  alarm. The  above  faults  need  to  be  photographed  or  video  sent  to  the  manufacturer  for judgment.

驱动器指示灯报故障

下表为指示灯正常工作时，在驱动器不同状态下的显示方式，可对照下表确认驱动器的当前工作状态。

| 停机 | 正常：绿灯常亮，红灯常灭 |
| --- | --- |
|  | 故障：红灯、绿灯交替闪烁指示故障代码 |
| 运行 | 正常：绿灯闪烁、红灯常灭 |
|  | 限流：绿灯闪烁、红灯闪烁 |
|  | 刹车：绿灯常亮、红灯快速闪烁 |

9电器原理图/Electrical      principle

继电器

Intermediate relay

RJ25-CL-D24

Red

漏电保护器

leakage   protector

急停按钮

传感器

开关电源

Switching Mode Power Suppl SE-450-24

24V

Blue       Brown

指示灯

Indicateor   light YW1P-1BUQ4G

24V

直流电机驱动器

DC motor driver

C20-400LR

W                        5V     HW         HV      HU     OV

Green      Yellow      Red      Blue    Green       Yellow

Z6BLD200-24GU-30S

DC GEAR MOTOR

直流减速电动机

电磁阀

Solenoid valve 4V210-08-24V

Proximity sensor TL-Q5MC1-Z

开关

Switch

YW1S-2E10

Black

Black

Black

8

4

| 环保清单一全自动蛋清蛋黄分离机 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 类别 | 部件名称 | 有害物质 |  |  |  |  |  |
|  |  | 铅 (Pb) | 汞 (Hg) | 镉 (Cd) | 六价铬 (Cr(VI)) | 多溴联苯 (PBB) | 多溴二苯醚汞 (PBDE) |
| 五金 | 主体 | O | O | O | O | O | O |
|  | 外壳 | O | O | O | O | O | O |
|  | 接蛋液托盘 | O | O | O | O | O | O |
|  | 蛋液导流槽 | O | O | C | O | O | O |
|  | 蛋清蛋黄分离器 | O |  |  | O | O | O |
|  | 托盘 | O |  |  | O | ○ | O |
|  | 螺钉 | O |  |  | O | O | O |
| 电器 | 开关 | O |  |  | O | × | × |
|  | 电源线 | O |  |  | O | O | O |
| 橡胶 | 密封条 | O |  |  | O | O | O |
|  | 密封圈 | O |  | ○ | O | O | O |
| 包装 | 包装箱 | O |  | O | O | O | O |
|  | 名牌 | O |  | O | O | O | O |
|  | 说明书 | O | O | O | O | O | O |
|  | 塑料袋 | O | O | O | O | O | O |
| 本表格依据SJ/T11364的规定编制。 b:表示该有毒有害物质在该部件所有均匀材料中的含量均在GB/T26572规定的限量要求以下。 区：表示该有毒有害物质至少在该部件的某一项均质材料中的含量超出GB/T26572规定的限量要求。 |  |  |  |  |  |  |  |