# OPERATION INSTRUCTIONS

HORIZONTAL OINTMENT  FILLER SERIES

GFA-150\300\500\1000

Please operate in accordance with the instructions strictly.

# CONTENTS

1.

1 ⸠1

1

1

2.

2 ⸠1

2

2

2

3.

4.

4 ⸠1

4 ⸠1.

4 ⸠1.

4 ⸠1.

4 ⸠1.

4 ⸠1.

4

4

4

Overview

Application

Model Explanation

Performance Characteristics

Safety Precautions   	 Safety Warnings    	 Electrical Connection

Emergency Stop Switch

Maintenance

Technical Parameters

Operation Guidelines

Preparation for Startup

1Cleaning

2Oiling ofOil Atomizer

3Air-pressure Regulation

4Check the position

of rotary disk valve

5Connect to the hopper

Test Run

Run

Stop

1                           5.     Maintenance and Service                         5

1                           5 ⸠1   Replacement of Seal Ring                        5

1                          5    Maintenance of Air Supply Treatment Unit             5

1                          5    Consumables                                           5

1                          6.     Troubleshooting                                       6

1                          6.1   Failure of Cartridge Cylinder                   6

1                          6.2   Inaccurate Filler Volume                        6

1                          6.3   Failure of Inching Work                           6

2                          7.     Technical Drawings                                 6

2                           7 ⸠1   Three-view Drawing                                 6

2                           7    Electric Circuit Diagram                           7

2                           7    Gas Circuit Diagram                                7

2                          8.     Attached Fittings                                     8

2                          9.     Acceptance Certificate                             8

3                          10.   Warranty Card                                         8

3

3

4

4

4

# 1.

1 ⸠1

1

1

2.

2 ⸠1

2

2

Overview

Application

Model explanation

Performance characteristics

Safety Precautions

Safety warnings

Electrical

Connection

Emergency Stop Switch

DGF Series Pneumatic Filler is of wide applicability and can be used for dosing filling of creams and liquids (20-2000cP) in the fields of daily

chemistry, food and beverage, pharmacy, chemistry and so on.

Inquire the manufacturer about other uses. Any damage due to misuse is out of limitation of warranty of the manufacturer. Such risks shall be borne by the

user solely.

Strictly following the instruments is a part of operation requirements.

GFA     -   300

Max. filling volume

GFA:standard

head

SGF:double

This machine is of reasonable structure, reliable performance and accurate   quantification. It applies pneumatically-driven structure. The shell and all the parts connected with filling materials are made of stainless steel (SS304 is

used for standard ones and SS316 can also be customized). The inner walls of

main parts are mirror-polished in roughness Ra≤0.2-0.4µm, which conforms

to  the   requirements     of  GMP   certification. Therefore,         it   is   rustless    and

corrosion resistant. In addition, the user can adjust the filling volume and speed within the type limitation as required.

Violation of these safety warnings and precautions may result in serious

personal injury and property losses. Qualified operators shall be familiar with all the safety warnings and precautions in the instructions. Correct handling  and fixing and careful test run, all these are key elements to ensure safe

operation and prevent from failures. The following reference points out

security-related operations in the instructions in particular. A corresponding icon is marked next to each reference.

The meaning of this icon: Attention!

“Attention” is to mark those activities or operations that have great impact on the normal operation of machine. Violation of these precautions may bring

damage to machine.

The meaning of this icon: Warning!

“Warning” is to mark those activities and operations that are harmful to people and property if the machine is not operated correctly.

During electrical operation, some parts involve voltages that threatening the personal safety. The electrical system or equipment should be operated by a skilled electrical technician or trained personnel operate it under experienced technicians' control and surveillance and also they must comply with the

corresponding electrical engineering regulations.

Attention: This series of products shall be mounted with safe and reliable leakage protection devices.

You should get quite familiar with the location of emergency stop button.

There is a red mushroom emergency stop button close to discharge nozzle.

Under extraordinary emergency conditions,  you should press the nearest Emergency Stop button and cut off both power and gas supplies, then the machine will stop immediately, in order to avoid more serious damage.

1

# 2   Maintenance              During maintenance, you must comply with the maintenance precautions

strictly, otherwise the safety operation of the machine can not be guaranteed.

3.  Technical Parameters

The following table takes standard single head horizontal          ointment  filler for example:

| Type | GFA-150 | GFA-300 | GFA-500 | GFA-1000 |
| --- | --- | --- | --- | --- |
| Filling volume (ml) | 10－150 | 20－300 | 20－500 | 30－1000 |
| Filling accuracy (FS) | ±0.5% | ±0.5% | ±0.5% | ±0.5% |
| Production capacity (cycle/min) | 1-25 | 1-25 | 1-25 | 1-25 |
| Applicable tolerance (m³/min) | ≥0.1 | ≥0.1 | ≥0.1 | ≥0.1 |
| Applicable pressure(Mpa) | 0.4-0.9 | 0.4-0.9 | 0.4-0.9 | 0.4-0.9 |
| Exterior sizes (mm) | 900×280×650 | 900×280×650 | 900×280×650 | 1000×280×650 |
| Net weight (kg) | 40 | 40 | 40 | 45 |

4. Operation guidelines

4 ⸠1

Preparation for Startup

4 ⸠1.1Cleaning

This machine uses three-strand power plug, so you should check new power socket to ensure its reliable grounding.

This machine shall be used with air compressor, of which, the air displacement shall be not less than 0.1m³/min and the applicable air pressure is able to adjust from 0.4 to 0.9 Mpa.

Before startup, clean all the parts which come into contact with the materials. The application of universal snaps and yokes in international type facilitates the loading and unloading.

4 ⸠1.2Oiling ofOil Atomizer

Check whether the oil cup (oil feeder) of oil atomizer is filled with sufficient oil. The oil atomizer shall be filled with 2.5 -7ºE lubricant (20# or 30# turbine oil). Oiling and volume regulation ways are listed as below:

Before cutting off the gas source, push upward or transverse by hand the water outlet at the bottom of filler to discharge the remaining gas. Screw out the

screw plug on the oil way in the center above the oil atomizer. Then oil may be injected through this hole. A vent hole, which is close to this oil way, may be  opened if it is difficult in oiling. The oil level shall be between the min. and

max. oil scale. Screw down each screw plug after oiling. A volume regulator inside above the filter can be used to adjust the volume of lubricant after

entering post pneumatic components. Generally, turn on the regulator in 1 to 2 turnings in close condition.

# 4 ⸠1.3Air Pressure Regulation

4 ⸠1.4Check the position of rotary disk valve

Refer to pressure regulation way as below:

(See Fig. 1 Air supply treatment diad) Pull up the pressure regulator button of air supply treatment device. Turn it slowly and observe the pressure gauge

finger. Press the button down when the finger points to the required air

pressure value. The air pressure shall be regulated accurately. If the air

pressure is lower than 0.4 Mp, the pneumatic system can not go on work   normally. If the air pressure is over high, it is possible that shock will occur and corresponding parts will be damaged earlier.

Fig. 1 Air supply treatment diad

Oil way

Air pressure regulator button

Air pressure gauge

Oil atomizer

Water filter

Water outlet

The core of rotary disk valve (Fig. 3-11) has been adjusted in the factory. But fortuitous event will also cause error position of valve core. Check whether   the valve core hole aligns with the valve body hole as shown in Fig 2 rotary disk before use. When adjusting to suction position, the rotary disk valve

cylinder (Fig. 3 -15) shall be in minimal position (close position). When

adjusting    to   discharge    position,     it   shall    be   in   maximal    position    (open position).

Fig. 2 Rotary Disk Valve                   Suction Position                       Discharge Position

A

Connect to hopper

A LS

Connect to cartridge

Connect to discharge nozzle

4 ⸠1.5Connect to the hopper

See Fig. 3 Whole Machine Parts. Generally, the hopper (Fig. 3-16) is fixed to the upper port of the rotary disk valve firmly. You can also choose stainless  steel tube adapter for pipe connection.

If it is necessary to heat, pressurize or mix for filling of material in large viscosity and poor fluidity, heating, pressurizing or mixing hoppers are  available for option. If the user refits the hoppers or other fittings with   permission or use them from other factory, the manufacturer will take no responsibility. Such risks will be undertaken by the user solely.

4   Test Run

4  Run

4  Stop

See Fig. 3 Whole Machine Parts. After getting ready of the preparation work

before    startup,    connect    the   “Inching     Switch”    (attached    fittings)     to   the corresponding jack (Fig. 3-5). Place the “Inching/Auto Option Switch” (Fig.

3-4) at the inching state, and then turn on the power on/off (Fig. 3-3) and check and ensure the Emergency Stop Switch (Fig. 13) is in normal state (the

machine will enter emergency stop state if press it down). Feed the material to the hopper or open the feeding pipe valve. Screw down all the connecting

yokes and start press the “Inching Switch” for test run.

Adjust the fore and back the throttles on the “Cartridge Cylinder” (Fig. 3-14) to control the feeding and discharge speeds properly. Shake “filling volume  regulation hand wheel” (Fig. 3-6) to keep the “Magnetic Switch (back)” (Fig.

3-7) move forward and backward to adjust the filling volume as required. And then lock “fastening screw” (Fig. 3-8).

Test whether repeated accuracy of filling volume is up to standard and check whether the discharge nozzle leaks. Put into normal work if there is no

abnormal condition. The normal work may be done in inching or automatic way (place the “Inching/Auto Option Switch” (Fig. 3-4) at automatic state).

Pay attention to properly adjust the interval time when the machine works in automatic state.

After the machine stops, you MUST turn off the power on/off, pull out the power plug and turn off the air supply valve. It is recommended to clean the material tubes, discharge nozzle,cartridge before off duty.

Fig. 3 Whole Machine Parts

# 16

13

10

14

6

12

11

15

9

8

# 1

5

4

# 2

7

3

1.Air supply treatment diad

2.Power socket

3.Power on/off

4.Inching/auto option switch

5.Inching switch jack

6.Filling volume regulation hand wheel

7.Magnetic switch (back)

8.Fastening screw

9.Piston

10.Cartridge

11.rotary disk valve

12.Discharge nozzle

13.Emergency stop switch

14.Cartridge cylinder

15.Rotary  disk  cylinder

16.Hopper

# 5.

and Service

Maintenance

Following the use of machine, the seal rings of each component will get worn, which will bring adverse effect on normal work. Replace the seal rings in time if there is any drip or leakage. DO NOT replace the seal rings until the machine is in stop state.

5 ⸠1

Replacement of Seal Ring

1.Seal ring on the piston (Fig. 3-9)

Loosen pressure screw on the cartridge (Fig. 3-10) and separate the cartridge from the piston in horizontal direction. Now replace with a new seal ring.

Apply a little lubricant oil to the inside wall of the cartridge and slightly push it in to prevent from the scuffing of seal ring. Align with the cartridge, half   lock the pressure screw and then start up the machine in the inching way till  the cylinder and cartridge are in right alignment. At last, lock the pressure

screw.

discharge nozzle (See

Discharge Nozzle

2.  Anti-drip seal ring on the                                     Fig. 4

Diagram)

out the

Screw out the “discharge port”, loosen “lock nut” and then slightly pull “discharge piston shaft”. Now replace with a new anti-drip seal ring.

Fig. 4 Discharge Nozzle Discharge

Fastening screw for discharge cylinder

Discharge cylinder

Lock nut

Discharge cylinder post

End cover

End face seal

ring

Seal ring for discharge piston shaft

nozzle

seat

Discharge

Discharge piston shaft

Anti-drip seal ring Discharge nosetip

Seal ring for discharge nosetip

changeable nozzle

Seal ring for

Changeeable nozzle

5

Air Source Treatment Unit

Filter core of air supply filter and water cup shall be cleaned regularly. The filter core shall be washed in the mineral oil and blow with compressed air.

Discharge the water in the filter by hand regularly. Ensure the sufficient oil in  the oil atomizer by regularly checking (Oil shall be added to the 2/3 level of oil cup)

works

the machine

consumables

timely.

5

Consumables

To

guarantee

spare

normally, maintain

# 6.   Troubleshooting

6.1   Failure of Cartridge Cylinder

|  | 1. Check whether air supply is connected well. 2. Check whether air pressure is too low. If so, adjust the pressure relief valve to proper air pressure. 3.Check whether the magnetic switch sends out signal. 4.Check whether the solenoid valve changeetdirection normally. 5. Check whether the throttle valve of cartridge cylinder is normal. If not, adjust the air flow properly. 6. Check whether there is any foreign object blocked in the discharge port and tubes. |  |
| --- | --- | --- |
| 6.2 Inaccurate Filler Volume |  |  |
|  | 1. Check whether the seal ring is worn (Refer to paragraph 5.1). 2. Check whether the magnetic switch (back) sends out signal. 3. Check whether the cartridge and cylinder is in right alignment when you disassemble or mount the cartridge. 4. Check whether there is any material leakage on the connector of each part and discharge nozzle. 5. Check whether the turning valve is blocked by any foreign object. 6. Check whether the discharge nozzle cartridge and its cylinder work 7.Check the position of turning valve core and valve body (refer to Fig. 4.1.4) |  |
| 6.3 Failure of Inching Work | 1. Check whether the inching socket (Fig. 3-5) is loose and the wiring is firm or not. 2. Check whether the inching switch works normally. 3. Check whether the fore and back magnetic switches work normally. 4. Check whether the solenoid valve exchanges the direction normally. |  |
| 7. Technical Drawings 7 ⸠1 Three-view Drawing |  |  |

7   Electric Circuit Diagram

2; 3; 4; 5; A B. i C # L1 A Fuse Pedal Switch Emergency Stop Switch Magnetic switch (Fore) Inching /Auto Option Switch B Magnetic switch (Back) Solenoid valve coil 1 Solenoid valve coil 2 N PE. C. D. 2.

7  Gas Circuit Diagram

2; 3; 5 4; A. Rotary disk cylinder Cartridge cylinder Discharge cylinder Throttle Valve Solenoid Valve Air Supply Air Supply Treatment Diad. A; B. B; C. C D. 1; 2.

# 8. Attached Fittings

2 pcs

4 pcs 2 pcs 2 pcs 2 pcs 2 pcs

1 pc Each 1 pc

1 pc

1 strip

1 pc

1 copy

1 copy

1 copy

1. Seal ring for discharge piston shaft

2. Anti-drip seal ring

3. Seal ring for discharge end

4. Seal ring for changeable nozzle

5. Piston seal ring

6. Yoke seal ring

7. Phillips screwdriver

8. Allen key (6, 8)

9. Open end wrench

10. Power wire

11. Pedal switch

12. Instructions

13. Certificate of Conformity

14. Warranty Card

9. Certificate of Conformity

CERTIFICATE  OF  QUALITY

Description:                                                Model:

Production No.:  	 Inspector:      	 Production Date:

The product is found eligible and permitted to leave the factory.

Certificate of Conformity

Description:                                                   Name of client:

Model:                                                            Address:

Production No.:                                              Dealer:

Production Date:                                             Purchase date:

10. Warranty Card

# 2008.04